Lab 7: ICMP

University of Windsor

Department of Electrical and Computer Engineering

ELEC 8560 – Computer Networks

Semester: Fall 2023

Student Name: Amey Mahendra Thakur

Student number: 110107589

Answers:

- The host's IP is 192.168.2.20, and its destination is 142.251.41.36.

- The ICMP packet specifies Type 8 and Code 0, accompanied by specific fields for checksum, checksum status, Identifier (BE), Identifier (LE), Sequence number (BE), and Sequence number (LE), with each field occupying 2 bytes.

- The ICMP packet includes Type 0 and Code 0, alongside fields for checksum, checksum status, Identifier (BE), Identifier (LE), Sequence number (BE), and Sequence number (LE), each field being 2 bytes in size.

- My host's IP address is 192.168.2.20, and the destination host's IP address is 172.217.1.4.

- If ICMP were to transmit UDP packets instead, the IP protocol number wouldn't be 01 but 0x11.

- The ICMP echo packet and the ICMP ping query packet share the same fields.

- The ICMP error packet comprises the header and the initial 8 bytes of the ICMP error packet, alongside IPV4 fields within the error packet.

- Error packets have a "type" field of 11, while the last three replies have a "type" field of 0. This distinction arises from the datagrams reaching the destination before the TTL expiration.